;

; B5IM-1.DOC  -  Adapts all modems using Hayes protocol to BYE5

;  03/03/86

;

;			      B5-IM

;			by Wayne Masters

;

;	For BYE500 (or higher) and intelligent modems like:

;

;		 Hayes Smartmodem (300 or 1200 or 2400)

;		 Racal-Vadic Maxwell 1200V or 2400V

;		 U.S. Robotics (300 or 1200 or 2400)

;		 Promodem, Anchor and others

;

;	Send comments and updates to:

;	Wayne Masters, Potpourri, (408) 378-7474

;

;***********************************************************************

; 03/03/86   Updated to reflect BYE504 and later equates

; 07/17/85   New release for BYE5

;

;***********************************************************************

;

;

; This file should work with any modem (300 or 1200 or 2400) that uses

; the "Hayes" compatible command and response codes.

;

; The code for this routine is resident in BYE5nn.  To invoke it, set

; B5IM EQU YES in the BYE5 equate table and follow these instructions.

;

; Your computer-to-modem interface should be wired so that:

;	- Carrier status from the modem is monitored from pin 8

;	- Your computer controls DTR to the modem on pin 20.

;	  (The USR S-100 does not require a wired interface.

;	   Install B5USR-n.INS as your Port I/O insert).

;

; Preinstallation instructions:

; ----------------------------

;	Anchor Mark XII

;		The Mark XII will not support DTR.  BYE5 will use the +++ATH

;		sequence to hang up the phone.  This sequence is repeated as

;		long as a carrier is present.  Older Mark XII firmware has a

;		problem recognizing the +++ escape sequence if the caller is

;		at 300 bps.  Use your MDM7 or MEX program in terminal mode and

;		issue ATI1 command.  Jim Warner from Anchor Automation called

;		me and said your checksum should be 129, 158, 221 or 222.  If

;		not, you need updated firmware.  You may also notice that the

;		callers with the new 2400 bps modems may not succeed in using

;		your system due to the tighter frequency tolerences being used.

;		The Anchor 212A frequency control is marginal.

;

;	HAYES 1200 switches should be set as follows:

;		UP  - 1, 2, 4, 6, 7

;		DOWN- 3, 5, 8

;

;	HAYES 2400 No switches so you must use a program like MDM740 and

;			set the software options to default as follows:

;		AT&F&W c/r  (sets non-volatile memory to factory specs)

;		AT&C1&D2&W c/r

;			(The &W writes this to non-volatile memory so

;			when we send ATZ it initializes correctly)

;

;	Mercury modems- See the file in this .LBR MERCURY.DOC for a detailed

;			explanation of configuring Mercury modems for BYE5.

;

;	ProModem 1200 switches should be set as follows:

;		ON  - 1, 2, 3, 7, 9

;		OFF - 4, 5, 6, 8, 10

;		 These switch setting should work with both BYE5 and MDM7.

;		 You can set ECHO EQU YES with this switch configuration

;		 since the ProModem will echo correctly for all operations.

;		 Using ECHO will provide a more reliable initialization.

;

;	RACAL-VADIC MAXWELL 1200V AND 2400V

;		Works out of the box, no preinstallation required.

;

;	USR 300/1200 Password, S-100 and 212A.

;		Try assembling BYE5 with NOATA EQU NO.  If your modem will

;		not answer the phone when ATA is issued the set NOATA EQU YES.

;		The only problem with this is if the computer is sick, your

;		modem will still answer the phone.  You can call USR and get a

;		RMA number and have your modem firmware updated.

;

;	Password

;		Set all 4 switches up (off).

;

;	S-100

;		Install B5USR-n.INS

;

;	212A Auto-Dial

;		Set switch 2 down (off), 1,3,4 up (on).

;

;	USR Courier 2400 Switch settings

;		ON   2, 3, 5, 8, 9

;		OFF  1, 4, 6, 7, 10

;	Note: The Courier will work with ECHO like the Hayes and Vadic.  Set

;	      switch 4 off and ECHO EQU YES.  Gives more reliable operation.

;

; This routine:

; ------------

;	1) Initializes the modem (and does echo verification, if ECHO is YES)

;	2) Detects phone ring

;	3) Answers the phone

;	4) Detects the speed of the remote caller

;	5) Sets the local computer port to that speed

;	6) Greets the user without him pounding on the RET key

;	7) Prevents modem from answering phone if local computer is sick

;	8) Deinitializes the modem when exiting BYE5 via CTL-C

;

; This file contains:

; ------------------

;	IMRING	 Main driver and dispatcher

;	IMINIT	 Modem initialization routine

;	IMQUIT	 Modem deinitialization routine

;	IMHANG	 Uses +++ATH to hangup the phone (if NODTR is YES)

;

; Installation instructions:

; -------------------------

; 1) Install your Port dependent insert in BYE5.  Verify that your

;	MDINIT routine does not CALL SETnnnn (delete it).  BYE5

;	sets the speed based on HSnnnn equate.	Verify that your

;	MDINIT drops DTR for appx. 2 seconds, raises DTR then has:

;

;		 IF	IMODEM

;		CALL	IMINIT

;		 ENDIF		; IMODEM

;

;		RET

;

;

; 2) Set the following equates in BYE5 as indicated:

;

;	IMODEM	EQU	YES	; For all intelligent modems

;	B5IM	EQU	YES	; Invokes this routine

;	HSnnnn	EQU	YES	; Set only one for your max modem speed

;

; 3) Set the following equates (in BYE5):

;

;	DOATZ EQU NO	; Try NO first.  Will speed up system considerably

;	ECHO EQU YES 	; for Hayes, ProModem or Racal Vadic Maxwell modems

;			; Also works with the USR Courier.  NO for all others.

;	ANCHOR EQU YES	; For the Mark-XII, No for others

;	NOATA EQU YES	; For older USR modems, No for others

;	NODTR EQU YES	; For Anchor Mark-XII or some Ozzie-1's, No otherwise

;	OFFHK EQU YES	; YES will issue ATH1 (busy out your phone), NO will

;			; issue ATS0=0 (allows phone to ring but not answer)

;			; when you are using your computer locally.

;	SHORTB EQU NO	; Yes if your modem has a short command storage buffer

;			; like the MultiModem.  BYE5 will send two short

;			; command strings instead of one long string.  Yes

;			; will also take a little longer to initialize modems.

;

; 4) Set the other equates in BYE5 as you normally do.

;	(Set PRGRSS EQU YES, and B5IM will display the modem result codes and

;	 commands on your local terminal). Handy until you have it working.

;

;

; Result codes expected:

; ---------------------

;		  For any modem - HS300, HS1200 or HS2400

;

;  0 OK 	- dumped in the bit bucket by IMRING

;  1 CONNECT	- if HS300 or HS1200, calls SET300 - for HS2400 checks

;		  for 10 or 11 first

;  2 RING	- causes IMRING to answer the phone

;  3 NO CARRIER - IMRING resets the modem and waits for next ring.  Most

;		  modems wait 20-25 seconds for a carrier during which

;		  time you cannot do anything but wait for a result code

;  4 ERROR	- modem could not execute previous command(s).	B5IM (if

;		  PRGRSS is YES) informs the sysop of a modem error and trys

;		  again.  You likely have a problem with your modem cable or

;	          computer insert if you see this message.

;

;		  For HS1200 or HS2400 modems

;

;  5 CONNECT 1200  - IMRING calls SET1200

;

;		  For HS2400 modems

;

;  6 CONNECT 2400 -  IMRING calls SET2400  (used by some clones)

;  7 (not applicable)

;  8 (not applicable)

;  9 CONNECT 2400  -  IMRING calls SET2400  (used by some clones)

; 10 CONNECT 2400  -  IMRING calls SET2400  (standard "Hayes" code)

;

;=======================================================================

#############################################################################