;  NBYEMDM.DOC  --  Specifications for the NBYEMDM.INC file for NBYE.MAC

;

;			Gary Case

;			585 Big Sky Court

;			Colorado Springs, CO

;				       80919

;			(303) 599-0744

;

;

;******************************************************************************

;

;   NBYE.MAC requires that all modem interface code be located in a file

;   named NBYEMDM.INC.  The required routines in this module are documented

;   below.  The following conventions are used in this documentation:

;

;	Inputs/Outputs:	Any required input is shown as an input to the

;			required register, with the expected register result

;			shown exiting the register.  For example, the modem

;			output routine requires the character to send in Acc,

;			and must not destroy the character.  The routine

;			documentation shows this as:

;

;				Char to Send ==> Acc ==> Unchanged

;

;		Status:	Any required status flags will be explicitly listed

;			after the register input/output conventions.  The

;			zero flag is represented by Z (set) or NZ (not set),

;			and carry is represented by C (set) or NC (not set).

;

;     Use of Registers: All Registers except the Acc must be preserved unless

;			the register input/output section explicitly shows

;			that a particular register is changed.  If a routine

;			may change a register, the exit condition is shown as

;			three question marks.  For example, if the character

;			output routine was allowed to destroy the character,

;			the documentation would show this as:

;

;				Char to Send ==> Acc ==> ???

;

;

;

;	NOTE:	To separate physical port I/O from the modem management

;		code, a separate NBYEIO.INC file is included when assembling

;		NBYE except in the S100 modem card cases.  Any NBYEMDM.INC

;		files you write should follow this convention.  Please

;		document the requirements for the associated NBYEIO.INC file

;		at the top of your NBYEMDM.INC file, so someone with the same

;		modem but a different I/O card (or computer) can easily adapt

;		the insert for his system.

;

;

;******************************************************************************

;

;

MISTAT:		;Routine to return the current input status of the modem.

;

;	??? ==> Acc ==>    0 if no input ready

;			0FFH if a character is ready

;

;Exit Status:	 Z ==> No Input Ready

;		NZ ==> An Input Character is Ready

;

;

;

;

MOSTAT:		;Routine to return the current output status of the modem.

;

;Exit Status:	 Z ==> Modem is Not Ready for output

;		NZ ==> Modem is Ready for the next output char

;

;

;

;

MINP:		;Routine to input the next character from the modem.  If no

;data is currently available, wait until the next character

;is received.  If possible, check the USART status for framing,

;overrun, or parity errors, and if any are detected clear them

;and substitute a null for the errored character.

;

;	??? ==> Acc ==> The input character

;			(Null if any errors were detected)

;

;Exit Status:	None

;

;

;

;

MOUT:		;Routine to send a character to the modem.  Be sure the modem

;port is ready for the data before sending it.

;

;	Character to Send ==> Acc ==> Unchanged

;

;Exit Status:	None

;

;

;

;

MINIT:		;Modem initialization routine.  This routine must:

;

;	1)  Ensure the modem is on hook.

;	2)  Initialize the modem port baud rate as appropriate.

;	3)  Set up the modem to desired default parameters.

;	    (See comments for MRING below)

;

;This is always the first routine NBYE calls in the

;NBYEMDM.INC insert.  It is also called after each telephone

;call to hang up and reinitialize the modem.

;

;Exit Status:	None

;

;

;

;

MEXIT:		;Modem exit routine.  This routine is called when the SYSOP

;has elected to exit NBYE.  It should 'shut down' the modem,

;returning it to the desired default state.  This is always

;the last routine called by NBYE in the NBYEMDM.INC insert.

;

;Exit Status:	None

;

;

;

;

MRING:		;Modem Ring Check routine.  This routine returns the current

;ringing status of the telephone line.  If your modem can't

;detect the ring status, then the initialization routine

;should set the auto-answer mode for the modem and, when the

;modem has answered the phone, MRING should return a Connect

;status.

;

;Exit Status:	 Z and NC ==> Telephone Not Ringing

;		NZ and NC ==> Telephone Is Ringing

;			C ==> Already Connected

;

;

;

;

MANSWER:	;Modem Answer Routine.  This routine must answer the telephone

;and wait for the carrier.  The wait time can be whatever you

;want, but 30 seconds is suggested as reasonable.  This routine

;is not called if MRING returns a connect status; however, it

;will be called if NBYE is invoked with a /A or /C option.

;

;Exit Status:	 Z ==> Didn't get a carrier

;		NZ ==> Connected with a carrier

;

;

;

;

MBAUD:		;Routine to return the connection baud rate.  If the modem

;supports more than one baud rate, this routine should

;determine the current baud rate (by waiting for a carriage

;return, line feed, ctrl-c, etc.) and return the baud rate

;indicator as shown.  If the modem always operates at the

;same baud rate, the routine can immediately return with the

;appropriate indicator.  If an input is necessary to determine

;the baud rate, this routine should wait a limited time

;(suggested 5 seconds) and return undetermined status if

;no input is received.

;

;	??? ==> Acc ==> Baud Rate Indicator

;

;			    0 ==>   110

;			    1 ==>   300

;			    2 ==>   450

;			    3 ==>   600

;			    4 ==>   710

;			    5 ==>  1200

;			    6 ==>  2400

;			    7 ==>  9600

;			    8 ==> 19200

;

;Exit Status:	 C ==> Couldn't Determine the Baud Rate

;		NC ==> Baud Rate Indicator is Set

;

;

;

;

MCARRIER:	;Carrier Check Routine.  Returns current carrier status.

;

;Exit Status:	 Z ==> No Carrier

;		NZ ==> Carrier Active

;

;

;

;

DELAY:		;Routine to delay 1/10 second.  This routine is placed here

;because:

;	     1) It may use some of the modem hardware (PMMI)

;

;	     2) Some modems (e.g. Hayes Smartmodems) provide

;		status information via response strings that

;		might be missed if we didn't keep polling

;		the modem port during delays.  In these

;		cases the delay routine can include a poll

;		to the modem port and can factor in the poll

;		time in the delay loop.

;

;Exit Status:	None

;

;

;

;

KDELAY:		;Routine to delay 1 millisecond.  This routine is placed in

;this insert for the reasons stated at DELAY.

;

;Exit Status:	None

;

;

;

;	***  End of Required Modem Routines  ***

;

;******************************************************************************

####################################################################