- HARDWARE CONFIGURATION -

3.0	HARDWARE CONFIGURATION

This section will assist you in choosing the correct computer equipment for use with C-NET v7.0 BBS.

3.1	MODEMS

There are two basic types of modems that are compatible with the program; User Port modems and modems that can be used with a Swiftlink/Turbo232 cartridge.  Of the User Port modems there are 3 different types that are compatible:

1650 modem and anything compatible (Westridge, Video 7, Total Telecommunications)

Mitey Mo and anything compatible (Hesmodem II)

Hayes Smartmodem and anything compatible (Prometheus ProModem, Commodore 1670, Linker, US Robotics Courier, Supra Modem 2400).  1200 and 2400 baud may be used with Hayes compatible modems only.

A modem is considered ‘Hayes compatible’ only if it understands basic ‘AT’ commands such as ATV0, ATH, ATS0=x, ATS7=x, ATX1, and ATA, RS-232 interfaces, which are necessary to connect most Hayes compatible modems to the Commodore 128, must be tested separately for their compatibility with C-NET 128.  RS-232 interfaces that have been successfully tested include the MSD and the Omnitronix brands.

3.2	FLOPPY DISK DRIVES

Generally, any disk drive that can be connected to your 128 and used normally with SAVE, LOAD, and OPEN commands, can also be used with C-NET v7.0.  Disk drives that have been successfully tested in the past include the 1541, 1571, SFD-1001, MSD-d2, Indus GT, PET 4040, 8050, 8250, 9060, 9090.  IEEE interfaces, which are necessary to connect SFD-1001’s and other parallel disk drives must be tested separately for compatibility.  The only IEEE interface that has been successfully tested is the Skyles Quicksilver 128.  There are two terms, however that you must be familiar with when configuring your C-NET v7.0 BBS.  When reference is made to a disk drive DEVICE, it is referring to the unit as a whole.  A disk drive may have one or two openings each referred to as a DRIVE.  A brand-new disk drive has a DEVICE number of 8.  If you plan to connect more than one disk drive to the 128 for use with the PROGRAM, you must insure that each disk drive has a unique DEVICE number (see disk drive instructions for information about changing the device number of a unit).  A disk drive with one disk capability is said to be a DRIVE 0.  With a two-disk drive unit, the individual disk units are numbered 0 and 1 (see disk drive instructions to which slots are which).

3.3	HARD DISK DRIVES

Hard drive manufacturers have recently discovered the enormous market potential that exists among Commodore computer users, primarily because of the bulletin board systems that they operate.  At the time of this publication, we have used two different hard drives, the first, and the one used the longest by the Author, is the Lt. Kernel hard drive.  Also, successfully tested is the CMD hard drive.  Before you purchase a hard drive, please contact us first for the latest compatibility information.

When configuring your hard drive, you may configure your hard drive any way you wish, however, it is suggested that you make at least 3 LU’s (partitions), (0 for system disk, 1 and 2 for whatever else you wish to use them for.  It is also suggested that you reserve at least 15,000 to 20,000 for LU 0.  This will leave you enough room for al the system files, your message bases (both local and network), as well as all the P-files (on-line games) that you wish to use).

3.4	VIDEO OUTPUT

C-NET uses the 80-column video chip exclusively for screen output.  You MUST be using and RGB monitor or a specially created MONOCHROME (one color only) cable plugged into the RGB output driving a composite monitor.  This cable may be made by connecting pin 7 of the RGB output port to the tip of an RCA connector and pin 1 to the ground of the same plug.  Also, the 40/80-column key at the top of the 128’s keyboard must be depressed at all times.

3.5	CARTRIDGES

No form of ‘FAST LOAD, MACH128, etc. or other types of software cartridge can be successfully used with C-NET 128 v7.0, but will only interfere and cause numerous problems.