DigiCHANNEL PC/X* DOS DEVICE DRIVER EXTENDED INT 14H FUNCTIONS

The following information is a list of the interrupt 14h functions that the

DigiCHANNEL PC/Xa DOS driver provides and their entry parameters. These funct-

ions are compatable with the ROM BIOS interrupt 14h functions to support the

DigiChannel PC/X*.

The entry parameters specified below are those to be used when requesting

one of the functions from an assembly or high level language program. It

should be noted that these entry parameters are not the same parameters found

in the XAPCM232.ASM driver listing. Those shown in the source listing are the

parameters used within the driver and not those passed to the driver in the

INT 14H call.

FNCT 0H  - INITIALIZE A PORT

ENTRY     AH : 0H

AL : PORT PARAMETERS

76543210

||||||``- data bits   : 0=5bits, 1= 6, 2=7, 3=8

|||||`--- stop bits   : 0=1 stop, 1=2 stop

|||``---- parity bits : 0=no parity, 1=odd. 3=even

```------ baud rate   : 0=110,  1=150,  2=300,  3=600

4=1200, 5=2400, 6=4800, 7=9600

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 0FFH ON ERROR, OTHERWISE

AH : 8250 FORMATTED LINE STATUS

AL : 8250 FORMATTED MODEM STATUS

FNCT 1H  - OUTPUT A CHARACTER

ENTRY:    AH : 01H

AL : CHARACTER

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 80H ON TIME OUT (30 SECS), OTHERWISE

AH : 8250 FORMATTED LINE STATUS

AL : CHARACTER SENT

FNCT 2H  - INPUT A CHARACTER

ENTRY     AH : 02H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 8250 FORMATTED LINE STATUS

AL : RECEIVED CHARACTER

FNCT 3H  - RETURN PORT STATUS

ENTRY     AH : 03H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 8250 FORMATTED LINE STATUS

AL : 8250 FORMATTED MODEM STATUS

FNCT 4H  - CHANGE BAUD RATE

ENTRY     AH : 04H

AL : PORT PARAMETERS

76543210

||||||``- data bits   : 0=5bits, 1= 6, 2=7, 3=8

|||||`--- stop bits   : 0=1 stop, 1=2 stop

|||``---- parity bits : 0=no parity, 1=odd. 3=even

```------ not used

BX : BAUD RATE IN DECIMAL  (e.g. 19200)

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 0 NO ERROR

AH : 0FFH FOR ERROR

FNCT 5H  - CHANGE PROTOCOL

ENTRY     AH : 05H, AL=PROTOCOL SELECTION CODE

AL : 0 = NO PROTOCOL

: PROTOCOL

7 6 5 4 3 2 1 0

| | | | | | | |

| | | | | | `-`-- XON/XOFF

| | | | | `------ DSR (Tx flow cntrl)

| | | | `-------- RTS (Rx) / CTS (Tx)

`-`-`-`---------- NOT USED

BL : NEW XON (IF 0, USE CURRENT VALUE)

BH : NEW XOFF (IF 0, USE CURRENT VALUE)

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 0FFH FOR CCB TIMEOUT

AH : 0 FCT GOOD

FNCT 8H  - ALTERNATE STATUS CHECK

ENTRY     AH : 08H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 8250 FORMATTED LINE STATUS

ZF : 0  THEN AL : CHAR IN QUEUE

ZF : 1  THEN NO CHAR QUEUED

FNCT 9H  - CLEAR BUFFERS

ENTRY     AH : 09H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 0 - FCT GOOD

AH : 0FFH - FCT BAD (CCB TIMEOUT ERR)

FNCT 0AH - INPUT QUEUE CHECK

ENTRY     AH : 0AH

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AX : NUMBER OF QUEUED CHARACTERS

FNCT 0DH - GET POINTER TO CH_KEY_RDY FLAG

ENTRY     AH : 0DH

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   ES:BX POINTS TO THE BYTE FLAG

FLAG = 0    : RX BUFFER EMPTY

FLAG = 0FFH : CHAR AVIALABLE IN RX BUFFER

NOTE : INTERRUPTS MUST BE ENABLED FOR THE CH_KEY_RDY FLAG

INFORMATION TO BE VALID.

FNCT 0EH - WRITE STRING

ENTRY     AH : 0EH

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

CX : # OF CHARS IN STRING

ES : SEGMENT OF STRING

BX : OFFSET OF STRING

RETURNS  AX = # CHARS TRANSMITTED

ZF = 0 : NO ERRORS

ZF = 1 : IF ERRORS

FNCT 0FH  - READ STRING

ENTRY     AH : 0FH

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

CX : # OF CHARS TO READ

ES : SEGMENT OF CALLERS BUFFER

BX : OFFSET OF CALLERS BUFFER

RETURNS:  AX = # CHARS READ

ZF = 0 : NO ERRORS

ZF = 1 : IF ERRORS  (LINE STS OR WRONG # CHARS READ)

FNCT 10H - CLEAR RX BUFFER

ENTRY     AH : 10H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS:  AH : 0FFH FCT ERROR (CCB TIMEOUT)

AH : 0    FCT GOOD

FNCT 11H - CLEAR TX BUFFER

ENTRY     AH : 11H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS:  AH : 0FFH FCT ERROR (CCB TIMEOUT)

AH : 0    FCT GOOD

FNCT 12H - GET TX BUFFER FREE SPACE

ENTRY     AH : 12H

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AX : # 0F FREE BYTES

FNCT 16H - CCB COMMAND

ENTRY     AH : 16H

AL : CCB COMMAND NUMBER

(40H - 54H)

BL : BYTE 2

BH : BYTE 3

CL : BYTE 1 FOR ALL-CHNL FNCS(SEE NOTE)

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS:  AH = 0FFH FCT ERROR (CCB TIMEOUT)

AH = 0    FCT GOOD

NOTE : CCB FNCS 4EH & 4FH REQUIRE NO CHNL#, SINCE THEY OPERATE ON ALL

CHNLS, BUT DX MUST STILL CONTAIN A VALID CHNL FOR THE BOARD

TO PASS THE INT 14H VALID PORT CHECK. FOR THESE ALL-CHNL FNCS,

PUT BYTE1 PARAMS IN CL BEFORE CALLING INT 14H

FNCT 18H  -  SEND BIOS COMMAND

ENTRY     AH : 18H

ES : SEGMENT OF USERS 16 BYTE COMMAND STRING

BX : OFFSET OF USERS 16 BYTE COMMAND STRING

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : FCT 18H STATUS

0   : NO ERRORS

80H : TIMEOUT ERROR

AL : MAILBOX STATUS (BYTE 1)

0   : NO ERRORS

8XH : BIOS ERROR (MAILBOX BYTE 1 - SEE BIOS SPEC)

ZF : 0 : NO ERRORS

ZF : 1 : IF ERRORS  (ERROR IN EITHER STATUS BYTE)

16 BYTE MAILBOX STRING RETURNED TO ES:[BX]

FNCT 19H - ENABLE / DISABLE  SPECIAL CHARACTER INTERRUPT

ENTRY     AH : 19H

BL : 0    - DISABLE SPECIAL CHARACTER INTERRUPT

BH : 0FFH - ENABLE SPECIAL CHARACTER INTERRUPT

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   AH : 0    - FUNCTION GOOD

AL : 0FFH - FUNCTION FAILED

FNCT 1AH - GET POINTER TO SPECIAL CHARACTER FLAG/COUNTER

ENTRY     AH : 1AH

BX : 0 - RETURN POINTER TO SPECIAL CHARACTER FLAG (BYTE)

BX : 1 - RETURN POINTER TO SPECIAL CHARACTER COUNTER (WORD)

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   ES : BX - POINTS TO THE FLAG OR COUNTER.

NOTE : SPECIAL CHARACTER INTERRUPTS MUST BE ENABLED FOR THE

FLAGS AND COUNTERS TO PROVIDE VALID INFORMATION.

FLAG  = FFh if a Special Character is in the receive buffer.

= 0   No Special Character in buffer; count is invalid.

COUNT = The number of characters in the receive buffer, up to

and including the last received Special Character. This

count is not valid if the Special Character Flag is 0.