Ethernet Supplement

(DOC Revision 2)

---------------------

Copyright (C) Roland De Graaf 1998

All Rights Reserved

---------------

http://www.degraaf.net

http://www.ameritech.net/users/pppbbs/index.htm

http://pppbbs.dyndns.com

ftp://pppbbs.dyndns.com

telnet://pppbbs.dyndns.com

INTRODUCTION

------------

Ethernet is, perhaps, the most common of local area

computer-to-computer network systems ever devised.  It allows

computers to communicate with each other -- quickly --

for the purpose of sharing files, printers, and a myriad of

other services and custom applications.

For the most, the connection scheme you employ is irrelevent

to application software.  Whether you make the connection via

thin-ethernet (10BASE-2), thick-ethernet (10BASE-5 or AUI),

twisted-pair (10BASE-T), or some other mechanism, the basic data

format(s) is (are) the same.

NOTE

----

PPPBBS' Ethernet support is DISABLED in the unregistered

version of the program.  Do NOT attempt to enable the Ethernet

support without a PPPBBS registration key.

MODES

-----

PPPBBS can make use of your ethernet card either one of two ways:

(1) INTERNET-FEED mode

INTERNET ==T1==> CSU/DSU --> ROUTER --Ethernet-LAN---

In this mode, PPPBBS is getting its Internet access from

your new or existing LAN/router.  The PPPBBS Network Access

Server uses the Ethernet feed to provide Internet access to

your incoming PPP callers, instead of doing a PPP dial-up

to your ISP.

Other computers on your LAN and users on the Internet

can also access the built-in PPPBBS TCP/IP Server applications,

unless you've disabled them.

If your Internet access looks something like this:

INTERNET ==T1==> CSU/DSU --> ROUTER --Ethernet-LAN---|

|

|

More    <----|----------|----|

Computers      |          |

Computer    Computer

Then you can use PPPBBS as a software-based dial-up

Internet Network Access Server.  In INTERNET-FEED mode,

there is no technical limit to the number of computers

you could have on your LAN running PPPBBS at once, other

than the "hard" limits imposed by Ethernet and other

standards, such as maximum stations per segment and so on.

That means you can serve more than 8 nodes.

That also means you could "scale" each PPPBBS dial-up

network access server computer to serve as many nodes as it

could reasonably handle, using multiple computers to distribute

the load of serving many nodes.

(2) INTERNET-GATEWAY mode

In this mode, PPPBBS is using a PPP dial-up to gate

the Internet to your Ethernet LAN.  Most of us can't afford

a T1 or other high-speed connection to the Internet,  so it

is very useful to be able to use a PPP dial-up connection

to your ISP as a way to bring Internet access to your

home or business LAN.

INTERNET ==PPP==> PPPBBS Computer --Ethernet LAN---|

|

|

More    <----|----------|----|

Computers      |          |

Computer    Computer

Any computer on your LAN capable of TCP/IP over Ethernet

will be able to have full access to the Internet.  Windows 95,

for example, comes with TCP/IP networking, and works great

over an Ethernet LAN.  If you've never used TCP/IP over a LAN

before, but you have used Windows 95 Dial-Up Networking before,

then you are in for a treat.  If PPPBBS is already connected

to your ISP, there is no waiting around for dialing.  When

you feel like "surfing the 'net" you can just go over to a

computer on your LAN and start surfing.  And the speed is

excellent.  You'll be amazed at how snappy Internet access

is over a LAN, even when the source is PPP-fed.

Any computer on your LAN capable of TCP/IP over Ethernet

will also be able to have full access to the PPPBBS built-in

TCP/IP server applications.  Your web site, smtp server,

pop3 server, ftp server, and telnet server are capable of

being accessed by the Internet and the computers on your LAN.

One example of a benefit from this is that it will help make

web site design easier, since you can "preview" your work

using a web browser running on another computer (station)

as you are working incrementally on updating your HTML and

GIF files.  Another benefit is that the smtp and pop3

servers and the PPPBBS mail processor can form the heart

of your LAN's mail system, offering local (ie station-to-station)

email as well as offering Internet email, using industry-

standard, TCP/IP-based mail software.

WHAT'S NEEDED

-------------

1. For best possible performance with the Ethernet support

enabled, you'll need a fast computer.  At least 486/66

or faster.  A Pentium-class machine would be even better.

It is highly recommended that you use buffered serial ports

based on either the 16550A (built-in 16 byte FIFO buffers)

or 16650 (built-in 32 byte FIFO buffers) chips.

It is recommended that all of your serial ports be using

the better serial chips.  If using INTERNET-GATEWAY mode, at

the very least the serial port being used for the PPP dial-out

to your ISP should be a buffered chip.

Note: The serial ports that come built-in on newer 486 and

Pentium motherboards are almost always buffered 16550A type

serial port chips.  Serial ports on today's multiport cards

(4 ports or more) designed for BBS/host system use are also

almost always buffered chips.

2. Obviously, you'll need an Ethernet card for PPPBBS to use.

PPPBBS will need to have exclusive use of this card, so

if the computer you are running PPPBBS on already has an

Ethernet card with an active LAN connection, you'll need

to have TWO Ethernet cards in the computer.

Your other LAN software may not be allowed to "touch"

the Ethernet card PPPBBS is using.  Other LAN software

may make use of a different Ethernet card in the same

computer, though.

If you are running PPPBBS under Windows 95, then Windows 95

must NOT be allowed to know about the card that PPPBBS

is using.  Windows 95 may make use of a different Ethernet

card in the same computer, though.

Non-Plug & Play cards work with PPPBBS without any special

action.  If the card is Plug & Play, however, then you must

use the set-up program that came with the card to set the

card to JUMPERLESS mode, and you must configure the card's

parameters (base address, IRQ, so on) manually.  It cannot

be used by PPPBBS if it is set to PnP (Plug & Play) mode.

If you running a Plug & Play Pentium computer, then

you may need to use the BIOS set-up program to inform

the BIOS which IRQ is in use by the Ethernet card so

that it won't try to assign that IRQ to a PnP card.

Tip: If you have one PnP type card and one non-PnP

type card in the same computer, and you are running

PPPBBS under Windows 95, you may get the most out of

things if let Windows 95 use and Plug & Play the PnP

card, leaving the non-PnP card for PPPBBS exclusive use.

3. You'll need a packet driver for the Ethernet card PPPBBS

will be using.

A packet driver is a .COM file DOS TSR type device driver.

Once loaded, it provides a standard programming interface

for applications software, and it also handles the

specifics of the Ethernet hardware, so that the application

doesn't have to.  The packet driver specification was

developed by FTP Software, Inc., and is a freely available

and relatively straight-forward to program.

Each different type of Ethernet card utilitizes a packet

driver written specifically for it.  Most Ethernet cards

ever developed and marketed have packet drivers avaialble

for them.

Many LAN cards come with a packet driver right on the

diskette that accompanies the card.  You'll find it

tucked away in a sub-directory.  It usually is a .COM file.

If you don't have already have a packet driver, you can

download the Crynwr Packet Driver Collection from web and

ftp sites all over the place.  The Crynwr Packet Driver

Collection is a famous set of free packet drivers, with

the set of .COM files covering a wide range of network

cards from different manufacturers.  You only need

the one .COM file that drives your particular card,

and you are all set.

The command line syntax for the packet driver is fairly

standard:

<drivername>.com [options] <soft int> <hardware IRQ> <base address>

For PPPBBS, [options] will be either "-i" (DOS)

or "-i -w" (Windows 95).

<soft int> is always "0x69".

<hardware IRQ> and <base address>

are the Ethernet cards IRQ and

base address

Note: All cards will require at least a base address and

hardware IRQ.  There may be additional parameters,

such as DMA channel, depending on the type of card.

Consult the documentation for your particular

Ethernet card's packet driver if you need assistance.

Executing the packet driver's .COM file with no

parameters will usually display command line help.

4. You'll need PKTWEDGE.COM.

PKTWEDGE.COM is a small TSR, provided by the PPPBBS author,

that loads before PPPBBS.  It provides and manages the

Ethernet receive buffers.  You do NOT have to load

PKTWEDGE.COM if you are NOT using PPPBBS's Ethernet

support features.

Unlike the packet driver, PKTWEDGE.COM requires no

command line options.

5. If you're using PPPBBS in INTERNET-GATEWAY mode to bring

Internet access to your LAN, you'll need a pool of IP

addresses for the machines on the LAN to use.

These IP addresses can be real or fake, it doesn't

matter.  If you happen to own a block of real IP

addresses, then by all means, use them.

If not, then you should form a block of fake addresses

based on the network number of your ISP.  The network

number is typically the first three numbers in an IP

address.  To illustrate this with example:

If your PPPBBS IP address when you dial into your ISP is

222.45.100.150 (static IP) or somewhere in the range of

222.45.100.1 to 222.45.100.254 (dynamically assigned IP)

then the network number is 222.45.100.

Remember: This is just an example.  Your actual network

number will be different.

Using the first three numbers, you can add one more

number to make fake IP addresses.  It is recommended

you make fake addresses starting out at 240, counting

down, example:

222.45.100.240

222.45.100.239

222.45.100.238

222.45.100.237

.

.

.

Write down the list on a pad of paper, and check each

one off as you assign it to a computer on your LAN.

Remember: Each computer on your LAN that will be using

PPPBBS for Internet access must be assigned its own unique,

static IP address.  Each IP address must be unique within

the confines of your LAN.

PACKET DRIVER & PKTWEDGE SET-UP (DOS)

-------------------------------------

If you are running PPPBBS under DOS, it is probabily best

to load the packet driver and PKTWEDGE.COM TSR using your

AUTOEXEC.BAT file.  As example showing this, for an NE2000

network card set for base address 300 IRQ 12 would be:

NE2000.COM -i 0x69 12 0x300

PKTWEDGE.COM

PACKET DRIVER & PKTWEDGE SET-UP (WIN95)

---------------------------------------

If you are running PPPBBS under Windows 95, it is probabily

best to load the packet driver and PKTWEDGE.COM TSR in a batch

file with PPPBBS.  Here is an example "STARTBBS.BAT":

NE2000.COM -i -w 0x69 12 0x300

PKTWEDGE.COM

PPPBBS.EXE

Edit the Properties of PPPBBS.PIF to execute STARTBBS.BAT,

instead of running PPPBBS.EXE directly, and you are all set.

Alternatively, you could load the packet driver and PKTWEDGE.COM

as part of Windows 95 AUTOEXEC.BAT, but that approach is not

recommended, and should only be tried if the STARTBBS.BAT

method does not work for you.

Remember: Windows 95 cannot know about or try to use the Ethernet

card that PPPBBS (and the packet driver) are using.  If Windows 95

tries to access the same card that PPPBBS is using, PPPBBS

will not be able to perform its Ethernet-related functions.

TESTING WITH PKTTEST.EXE

------------------------

For your convenience, a simple "LAN Sniffer" has been provided

with PPPBBS.  PKTTEST.EXE is a simple packet driver-based LAN

activity display that displays on screen a "hex dump" of LAN data

traffic, and logs the displayed data to disk for possible analysis.

Like PPPBBS.EXE, PKTTEST.EXE expects the packet driver to be

loaded and set for <soft int> of 0x69.  When PKTTEST.EXE starts

up, it will display information about your packet driver, and

the Ethernet card's hardware Ethernet address, and it will start

displaying data traffic on the screen.  If you have a really busy

LAN, the information display may scroll off your screen rather

quickly.

You should run PKTTEST.EXE before you try PPPBBS.EXE, just

to make sure that things are working correctly, and that the

packet driver is indeed correctly receiving packets.  So long

as your LAN isn't dead-quiet, you should see *some* activity.

If the informational display looks OK, and data traffic is

being received and displayed on the screen, then this is a good

indication that the packet driver and your Ethernet card

are talking together correctly.  If the informational display

looks OK, but data traffic is not being received, it could be

an indication of a problem, or it could just be that nothing

is going on over your LAN at the moment.

Press [Esc] to exit PKTTEST.EXE.

WARNING: PKTTEST.EXE records all data it receives to a log file

PKT.LOG, which can grow quite large quite fast.  Do NOT leave

PKTTEST.EXE running unattended, or you may come back to your

computer to find your hard drive full.  You may freely delete

PKT.LOG after running PKTTEST.EXE.

For more PKTTEST.EXE information, see the sections on

TROUBLE-SHOOTING, and SUPPORT FOR NEW FRAME TYPES.

PPPCFG RELATED SET-UP (INTERNET-FEED MODE)

------------------------------------------

To set-up PPPBBS for INTERNET-FEED mode, you must set the

DialOut Configuration Port # to 9.  This tells PPPBBS not

to dial-out, but instead to use the Ethernet LAN for

its Internet feed.

PPPBBS also needs to know the Ethernet address of your

T1 or other hardware router.  Either you can specify

the Ethernet address of the hardware router directly

(if you know what it is), or you can specify the IP

address of the hardware router, and PPPBBS will try

to automatically learn the Ethernet address using ARP

each time it is executed.

Ethernet addresses should be entered in PPPCFG in

conventional aa-bb-cc-dd-ee-ff format.

IP addresses should be enter in PPPCFG in conventional

aaa.bbb.ccc.ddd format.

PPPCFG RELATED SET-UP (INTERNET-GATEWAY MODE)

---------------------------------------------

To set-up PPPBBS for INTERNET-GATEWAY mode, you must set

one of the Host Ports Port # to 9.  This tells PPPBBS to

use the Ethernet card for this channel.

Initially, you should also enable the PROXY ARP function.

SECURITY ISSUES / FIREWALL (INTERNET-GATEWAY MODE ONLY)

-------------------------------------------------------

In INTERNET-GATEWAY mode, you are using a PPP connection

to your ISP to give Internet connectivity to your LAN. It is

only normal to be concerned about security issues.

First, PPPBBS only "talks" TCP/IP, UDP/IP, and ARP over

the LAN.  It cannot (and will not) interact with other LAN

protocols (for example, NETBEUI, NetBIOS, IPX/SPX, Artisoft,

Novell, etc).  Therefore, it can only interact with other

LAN computers and LAN hardware that support TCP/IP, UDP/IP,

and ARP.

Second, users on the outside world (the Internet) will

not be able to access other TCP/IP hosts on your LAN without

specific action from you to make it possible.  Users on the

outside will be able to access the built-in PPPBBS servers,

if you haven't disabled them.

The FIREWALL effect is actually a natural side-effect of

PPPBBS's ability to be a network access server for your

PPP dial-ins and your LAN without requiring you to own a

block of IP addresses.

To make TCP/IP-based server applications running on other

TCP/IP (non-PPPBBS) computers on your LAN accessible to the

Internet, you must specifically enable it by editing/creating

ASCII file RESERVE2.CFG.  RESERVE2.CFG is just like RESERVED.CFG

(See PPPBBS.DOC) except that you specify the IP address of

the target computer instead of a user handle.

Note: If you shutdown and restart PPPBBS, before the target

computer can be communicated with, it must either be rebooted,

or perform some type of outbound net activity such as telnet

or ftp or whatever.  This is so that PPPBBS has a chance to

learn the other machine's Ethernet address via ARP.

SETTING UP MICROSOFT TCP/IP ON COMPUTERS WHICH WILL

ACCESS THE INTERNET THROUGH PPPBBS (INTERNET-GATEWAY MODE ONLY)

---------------------------------------------------------------

The point of INTERNET-GATEWAY mode is to bridge a PPP

Internet feed into your Ethernet LAN, so the Internet

can be accessed from more than one computer at your

your location, be it home or business usage.

This document section gives guidelines and shows an

example of how to set-up the other TCP/IP computers on

your LAN for Internet access through PPPBBS.

The computers on your LAN that use PPPBBS to access the

Internet will need to have some type of TCP/IP stack (also

called a TCP/IP protocol) installed and active.  There is

a TCP/IP stack available for most any computer platform,

except perhaps some of the "ancient" 8-bit computers.

Windows 95 comes with Microsoft's TCP/IP stack, and is

quite probabily one of the most-used versions of TCP/IP

today.  Since it is the most popular, it will be used

for this set-up example.  Set-up of other TCP/IP stacks

on the PC or other platforms will be similar, and the

same principles discussed here will apply.  TCP/IP

is TCP/IP afterall, and they are all supposed to work

the same way and need the same configuration information;

as a result configuring one TCP/IP is similar to configuring

most any other.

It is assumed you already have an Ethernet card installed

and Windows 95 is configured to recognize the Ethernet card.

For the purposes of this example, assume the Ethernet card is

an NE2000 compatible card.

Step 1. Open up the "Control Panel", and double-click "Network"

There should be entries that look like:

[Adapter]  NE2000 Compatible

[Protocol] TCPIP -> NE2000 Compatible

Note that [Adapter] and [Protocol] are actually shown

on the screen as little icons, and that there may

be other entries in the list.

If TCPIP -> NE2000 Compatible isn't displayed,

click "Add", click "Protocol", click "Add", click

"Microsoft", click "TCPIP", click "OK".

Step 2. Click "TCPIP -> NE2000 Compatible"  and click

"Properties"

IP Address Tab

--------------

Assign an IP address from the pool you made up

in "WHATS NEEDED" item 4.  The Subnet Mask should

be set to 255.255.255.0.

DNS Configuration Tab

---------------------

Enable DNS.

Windows 95 will insist you fill in the Host field, so

assign the machine a hostname of your own choosing.

Enter in the IP address(es) for your ISP's domain

name servers.  (Unless you run your own domain

name servers, in which case you could use those.)

Gateway Tab

-----------

Windows 95 handling and interpretation of this tab

seems to be somewhat of a mystery.

There are four possible ways to set-up this tab,

and you may need to experiment to find the exact

one which best fits your set-up and network needs.

Method #1

---------

Leave it at default setting (blank).  Make sure

you enable PPPBBS's PROXY ARP feature though.

If you have trouble with DNS look-ups, see Method #3.

Method #2

---------

If your PPP account with your ISP is static IP,

then you might be able to use method #2.  Put in

PPPBBS's IP address on this tab.  You might be able

to disable PPPBBS's PROXY ARP, and all should be OK.

If you have any difficulty, though, re-enable PROXY ARP.

If you have trouble with DNS look-ups, see Method #3.

Method #3

---------

If your ISP's domain name server's are on a different

network number than your own, then methods #1 and #2

might not work correctly for you in that everything

will work except domain name service requests.

If that is the case, then putting your ISP's domain

name server IP address here seems to fix the problem.

You will need to enable PPPBBS's PROXY ARP.

Method #4

---------

Method #4 is a combination of method #3 and #4.

Windows 95 lets you specify a list of several IP

addresses as possible gateways.  You can try listing

both the PPPBBS IP address and your ISP's domain

name server IP address, and you can also try them

in different order.

You may need to enable PPPBBS's PROXY ARP.

Bindings Tab

------------

This tab can be left at default settings.

Advanced Tab

------------

This tab can be left at default settings.

WINS Configuration Tab

----------------------

This tab can be left at default settings.

Step 3. Once all TCP/IP settings have been entered to your

satisfaction, click "OK" to save the changes.  Windows 95

may ask you to reboot the computer for the changes

to take effect.  When the computer is done rebooting,

it is ready to start accessing the Internet through

your Ethernet LAN.

Microsoft TCP/IP Set-Up Possible Q & A

--------------------------------------

Q: "If I set-up a computer that has been using Dial-Up

Networking to use the Internet, to access the Internet over

the LAN, how does that affect my previous Dial-Up Networking

settings and capabilities?"

A: If you open up the "Control Panel" and double-click "Network",

you might see something like:

[Client]   Client for Microsoft Networks

[Adapter]  Dial-Up Adapter

[Adapter]  NE2000 Compatible

[Protocol] TCPIP -> Dial-Up Adapter

[Protocol] TCPIP -> NE2000 Compatible

If you examine the "Properties" of TCPIP -> Dial-Up Adapter

and TCPIP -> NE2000 Compatible, you'll see that Windows 95

keeps a separate set of settings for each instance of TCP/IP.

Which set is active at any given moment in time, is dependant

on whether Dial-Up Networking is connected or not.  If

Dial-Up Networking is connected to your ISP, then it is

the active link for TCP/IP operations.  If you are not

currently connected with Dial-Up Networking, then the LAN

becomes the active link for TCP/IP operations.

Q: "I run PPPBBS under Windows 95.  Do I have to install the

Microsoft TCP/IP stack on my PPPBBS machine?"

A: If you happen to be running PPPBBS under Windows 95,

you DO NOT need to set-up Windows 95 TCP/IP on the

machine for PPPBBS to function correctly.

You MAY set-up the Microsoft Windows 95 TCP/IP stack,

if desired, so long as it is NOT bound to the Ethernet

card that PPPBBS is using.  It MAY be bound to a (second)

Ethernet card that PPPBBS is not using and/or to the

Dial-Up Adapter.  This means you could run "winsock"

applications on the same computer that is running PPPBBS.

Remember: Windows 95 can't even be allowed to "know"

about the Ethernet card that PPPBBS is using.  If

Windows 95 knew about the card, it would interfere

with PPPBBS ability to perform Ethernet functions.

TECHNICAL NOTES

---------------

ETHERNET FRAME TYPE

-------------------

Today's LAN cable is multi-lingual.  Depending on

what's running at the moment, there could be many

different Ethernet "frame types" in use.  While the

basics are the same, there are some variations and

possible deviations from the "standard format."

PPPBBS supports the Ethernet frame type "DIX Ethernet"

also called "Ethernet Bluebook."  This is the same

frame type as used by Windows 95 TCP/IP when it is

communicating over an Ethernet LAN.

If you are using PPPBBS in INTERNET-GATEWAY mode, and

if all of the computers on your LAN that will use Internet

access are running Windows 95's TCP/IP stack, then you

do not need to be concerned about the frame type.

PPPBBS is automatically compatible with Windows 95.

Computers not running Windows 95 may also be compatible,

if their TCP/IP stacks are communicating through the

Ethernet card using the DIX Ethernet protocol.

If you are using PPPBBS in INTERNET-FEED mode, then

frame type is something to think about.  In order for

PPPBBS to be able to talk to your Router, it must be

speaking the same frame type as the Router.  If your

Router supports DIX Ethernet or Ethernet Bluebook

or standard Ethernet, then PPPBBS should work for you

right off the bat.  If your Router speaks a different

format, then contact the PPPBBS author for possible

inclusion in PPPBBS.  You may be asked to supply

a small sample of the frames for the frame type

to be added.  (PKTTEST.EXE can help you with this.)

If you haven't purchased your Router yet, then you can

ask ahead of time what Ethernet frame type(s) it supports.

(And, if the salesman can't answer that question,

you might want to shop for your Router elsewhere.)

SUPPORT FOR NEW FRAME TYPES

---------------------------

If you are operating PPPBBS in INTERNET-FEED mode,

and find that your hardware router and PPPBBS are not

communicating correctly, it is possible that the two

entities are trying to use different Ethernet frame types.

To make a sample to send to technical support, follow

these steps:

1. Quiet your LAN as much as possible, with only

the hardware router talking

2. Delete PKT.LOG (to get a fresh start)

3. If you've not already done so, load the appropriate

packet driver for your Ethernet card, and run

PKTTEST.EXE.

4. Once PKTTEST.EXE has captured a dozen or so frames,

Press [Esc].

5. If you know what to look for, edit PKT.LOG to remove

any extraneous frames (ie frames that didn't come from

the hardware router).  If not, leave the file intact.

6. Mail or email the file PKT.LOG to PPPBBS Technical Support,

along with the following information (if known):

-Your Voice Phone Number/Hours Can Be Reached

-LAN type

-Hardware Router type/model/manufacturer/date purchased

-Other LAN software running on the LAN:

Novell

LANtastic

Microsoft Networking or Windows For Workgroups

Other

While it is very important that PPPBBS be as compatible

as possible with as many formats as possible, it is not

guaranteed that this procedure will provide enough

information to support your desired format, nor is it

guaranteed that support for new formats will be added.

GOOD NEIGHBOR

-------------

PPPBBS is a "good neighbor" on the LAN:

PPPBBS receives only packets destined for it and

broadcast packets

PPPBBS tolerates irrelevant broadcast packets from

other computers on the LAN

PPPBBS recognizes/responds only to IP and ARP protocols

(IP covers both TCP/IP and UDP/IP)

PPPBBS speaks only when spoken to

This means you can have many other computers and many

other types of frame types, protocols, and so on, going

on all at the same time on your LAN.

ARP

---

ARP is short for Address Resolution Protocol, and its

function is to help TCP/IP computers find each other on an

Ethernet LAN.

Each Ethernet card has been programmed at the factory with

a unique hardware Ethernet address.  This address is unique in

the world.  While TCP/IP computers might know each other's IP

addresses, they have no idea, at least initially, what another

computer's Ethernet address is.

When one TCP/IP machine on an Ethernet LAN wants to talk

to another, if it doesn't know the Ethernet address already,

it must dynamically "learn" this information, by sending

an ARP request.  ARP requests are broadcast packets that

basically say:

"I am looking for the station whose IP address is a.b.c.d,

and if you are a.b.c.d, please reply back with your Ethernet

address."

If the station with an IP address of a.b.c.d exists and is

running at the time, it will reply back, and the TCP/IP

connection can proceed.

PPPBBS can both generate and receive/reply to ARP requests.

PPPBBS generates ARP requests only when in INTERNET-FEED

mode, and then only when it needs to figure out the Ethernet

address of your Router.  If your Router is not capable of

responding to ARP, you can manually tell PPPBBS what your

Router's Ethernet address is.  The automatic method is

preferred, though.

PPPBBS receives/replies to ARP requests when in either

INTERNET-FEED or INTERNET-GATEWAY mode.

In INTERNET-FEED mode, it will only reply to queries about

its own IP address (configured with PPPCFG).

In INTERNET-GATEWAY mode, it will reply to its own IP address,

and if you have the PROXY ARP option enabled, it will also

reply to queries about other IP addresses, so long as that

IP address is not already listed in DONTARP.CFG.

DONTARP.CFG is an optional configuration file you create with

an ASCII text editor.  It lists out IP addresses, one per line,

of the other TCP/IP computers (hosts) on your LAN.  WARNING!

Do NOT put PPPBBS's IP address in this file! Do NOT put IP

addresses of system's out on the Internet in this file!

WHAT IS PROXY ARP?

PROXY ARP only applies to INTERNET-GATEWAY mode.

In INTERNET-GATEWAY mode, PPPBBS is acting as a gateway

to the Internet for your Ethernet LAN.  You could have

one or more computers on your LAN accessing the Internet

through your LAN at the same time.  When these computers

want to connect to a TCP/IP or UDP/IP server on your

LAN or on the Internet, they will automatically generate

ARP requests to learn what Ethernet station they need

to send the data to, to get the job done.

Computers on your LAN could reply back for themselves;

that is not a problem.  But what about systems ("hosts")

on the Internet, who are not connected to your LAN?

There is where PROXY ARP comes in.  If you've enabled

the feature, then PPPBBS, when in INTERNET-GATEWAY mode,

will respond to ARP queries, giving its own Ethernet

address in the reply, even if the IP address does not

match what you have configured for PPPBBS in PPPCFG.

This makes the set-up on the Windows 95 (TCP/IP) side

easier and more painless to use, and also makes things

much easier if you happen to be using a PPP account

with dynamic IP addressing, since the IP address

of PPPBBS will change when re-dialing your ISP.

If you disable PROXY ARP, then you must use the Windows

95 TCP/IP Properties "Gateways" tab to explicitly set

the IP address assigned to PPPBBS in PPPCFG as the first

address in the list of gateways.  You can see where a

having a static IP address for PPPBBS would almost be

a requirement if you disable PROXY ARP.

TROUBLE-SHOOTING

----------------

PPPBBS has been thoroughly tested to make sure that

the Ethernet support works, when set-up properly.  If things

are not set-up properly, the Ethernet support may still work

fully or partially or not work at all, and the worst part is

that the computer will not be able to diagnose the problem

for you.

To that end, check-lists and some questions and answers

have been put together in advance that should help you

pin-point most configuration errors, should you have

difficulty getting the Ethernet support working initially.

PPPBBS System Check List

(Both INTERNET-FEED & INTERNET-GATEWAY Modes)

---------------------------------------------

1. Is the packet driver loaded on the PPPBBS system?

Is the command line correct?

Is 0x69 specified as the <soft int>?

Is the proper <hardware IRQ> specified?

Is the proper <base address> specified?

Are there any additional parameters required,

such as <dma>?

Are hexadecimal values properly prefixed with "0x"

such as 0x300 (hexadecimal 300)? Example:

NE2000.COM -i -w 0x69 10 0x300

which would load the driver and configure it for

<soft int> 0x69 (hexadecimal),

<hardware interrupt> 10 (decimal), and

<base address> 0x300 (hexadecimal).

When the packet driver loads, does it report your

Ethernet address as 00-00-00-00-00-00 or FF-FF-FF-FF-FF-FF?

If so, you have the <base address> set wrong.

If the Ethernet address is reported as something other

than 00-00-00-00-00-00 or FF-FF-FF-FF-FF-FF, then you

probabily have the <base address> set correctly.

When you run PKTTEST.EXE, does it report your Ethernet

address as 00-00-00-00-00-00 or FF-FF-FF-FF-FF-FF?

If so, then possibly the packet driver's <soft int> is

set to something other than 0x69.

If you run PKTTEST.EXE, and it fails to see any data

traffic on your LAN:

You may just have a quiet LAN.  Try to make some

activity if you can. (You can always initiate some

activity by rebooting a Windows 95 machine which

has TCP/IP bound to a LAN adapter.)

-or-

The other computers on the LAN may be misconfigured,

and are therefore not sending data.

-or-

You may possibly have a <hardware interrupt> problem,

such as a hardware conflict (two devices trying to

use the same hardware interrupt) or the packet driver

may be misconfigured and trying to use the wrong IRQ.

Double-check that the <hardware interrupt> (IRQ)

in use by the Ethernet card is not in use by another

device, and that the packet driver is being told

to use the correct <hardware interrupt>.

2. Is PKTWEDGE.COM loaded?

3. Has PPPBBS been told to support Ethernet, either by

setting the DialOut Port # to 9 (for INTERNET-FEED mode)

or by setting one of the Host Ports Port # to 9

(for INTERNET-GATEWAY mode)?

4. If FEED mode, has PPPBBS been told either the IP address

or Ethernet address of your T1/Other Hardware Router?

5. If GATEWAY mode, check the PROXY ARP setting.  If it has

been disabled, and you are having difficulty, try enabling

PROXY ARP to see if the problem goes away.

non-PPPBBS System Check List (INTERNET-GATEWAY Mode Only)

---------------------------------------------------------

1. Is the Ethernet card installed and working?

If the computer is able to communicate with

computers other than PPPBBS, but can't access

PPPBBS or the Internet, then your Ethernet card

is probabily working, and the problem is elsewhere.

2. Is TCPIP installed, and is it bound to the Ethernet card?

If the answer is yes, but you are experiencing difficulty,

then double-check the TCPIP Properties for a possible

configuration error.  If that seems to check out OK,

then try a different "method" for the Gateway tab.

(See SETTING UP MICROSOFT TCP/IP ON COMPUTERS WHICH WILL

ACCESS THE INTERNET THROUGH PPPBBS).

3. If you are able to use TCP/IP applications if you specify

the IP address, but not if you specify hostname, you either

have a configuration error on the DNS tab, or you may need

to try a different "method" on the Gateway tab.

(See SETTING UP MICROSOFT TCP/IP ON COMPUTERS WHICH WILL

ACCESS THE INTERNET THROUGH PPPBBS).

Possible Q & A (INTERNET-GATEWAY MODE ONLY)

-------------------------------------------

Q: "I have everything working OK. But if I shutdown

PPPBBS and then restart it right away, my other

computers can't access the Internet for a few minutes."

A: This is an ARP "thing."  Once your computers have

learned PPPBBS's Ethernet address through ARP, they

retain the information for a period of time.  PPPBBS

does this too, but it does not retain the information

if you shut it down and restart it.  If you wait 4

minutes for ARP timers on the non-PPPBBS computer(s)

to timeout, or re-boot the non-PPPBBS computer(s),

everything will "magically" start working again.

PPPBBS was designed to be a program that runs 24 hours

a day, 7 days a week, so this problem doesn't come up

too often for most system operators.  If you want to

use PPPBBS "part-time" just for its PPP -> Ethernet

feature for your LAN, that's great too, just try to avoid

shutting down (and restarting) while other systems on

your LAN are potentially using it.

HARDWARE SET-UP TIP

-------------------

PPPBBS Ethernet support works best if your SERIAL PORTS

are on (a) higher-priority IRQ(s) than your ETHERNET CARD.

Here is the listed of IRQs in order from HIGHEST to LOWEST

priority:

IRQ             Usually assigned to

---             -------------------

IRQ0            System Timer

IRQ1            Keyboard

IRQ8         Real-Time Clock

IRQ9

IRQ10

IRQ11

IRQ12

IRQ13        Math Coprocessor

IRQ14        Primary IDE

IRQ15        Secondary IDE

IRQ3            COM2

IRQ4            COM1

IRQ5

IRQ6            Floppy Controller

IRQ7            LPT1

Amongst other things, this means that if you are using COM1 and

COM2 at their normal IRQ assignments, the best place for the

Ethernet card is IRQ 5.

If you are using a multi-serial board that supports IRQs

10, 11, or 12, then you have more flexibility.