ISDN Supplement

(DOC Revision 0)

---------------------

Copyright (C) Roland De Graaf 1998

All Rights Reserved

---------------

http://www.degraaf.net

http://www.ameritech.net/users/pppbbs/index.htm

http://pppbbs.dyndns.com

ftp://pppbbs.dyndns.com

telnet://pppbbs.dyndns.com

INTRODUCTION

------------

PPPBBS.DOC covers using PPPBBS over "regular" analog modems

and null-modems.  ETHERNET.DOC covers PPPBBS's Ethernet

connectivity capabilities.  This document covers yet another

type of connectivity -- ISDN.

WHAT IS ISDN?

-------------

In the simplest of terms, ISDN is a *digital* phone line.

Depending on where you live and the tariffs in your state,

ISDN is usually priced affordable to the average consumer.

It offers 64k bits per second or 128k bits per second digital

connections, with a substantially reduced error-rate compared

to analog modems over a regular analog phone line.

In addition to being able to make direct computer-to-computer

calls for Internet access and remote computer access, ISDN lines

can also place/receive analog (voice, fax, analog modem) phone

calls -- just like a regular phone line.

Your PPPBBS system can use ISDN to make the PPP connection

to the Internet just like it would use a regular modem.  You

could also support ISDN incoming calls as well.

If you plan on using PPPBBS to gate your "PPP Internet

Feed" to your Ethernet LAN, regular modems (33.6 and 56k)

work great, but PPP over ISDN is even better.

WHAT EQUIPMENT IS REQUIRED FOR ISDN?

------------------------------------

For each ISDN line you have installed, you'll need to have

an ISDN Terminal Adapter (T/A).  NOT ALL ISDN TERMINAL ADAPTERS

WILL WORK WITH PPPBBS -- SEE "PPP OVER ISDN" FOR THE REASON.

An *external* (versus internal) T/A is strongly recommended,

as most internal T/A's are not as capable as external units.

In general, an external ISDN T/A will plug into the ISDN line,

and the T/A will provide:

one DB25 (serial port) connector, which you connect to your

computer just as if the T/A were a "modem".

one or two RJ11 POTS ports (POTS=Plain Old Telephone Service),

which you can use to plug in one or two standard telephone

deivces, such as telephones, analog modems, fax machines,

answering machines, and so on.

WHAT EQUIPMENT IS RECOMMENDED?

------------------------------

At this time the only T/A that can be strongly recommended

is the US Robotics (3COM) Courier I-Modem.  It works well with

PPPBBS, seems to be relatively bug-free and reliable, and it

has many useful features usable to a PPPBBS operator, such

as a built-in V.Everything analog modem, X2 host support, etc.

Best of all, if you shop around you can get the US Robotics

(3COM) Courier I-Modem for around $270 -- about the average cost

for a T/A.

In theory you can use any ISDN T/A with PPPBBS, but the

US Robotics (3COM) Courier I-Modem is the only offically

supported ISDN T/A recognized by PPPBBS technical support.

PPP OVER ISDN

-------------

When using an ISDN T/A for Internet access, it is important

to program the T/A to be in PPP mode.  While in PPP mode,

any out-of-band data (ie non-PPP packets) or transmission

errors are automatically filtered out by the T/A.

PPP mode generally comes in three flavors:

1 Regular, Single-Link PPP    (64 kbps) (also called Synchronous PPP)

2 Compressed, Single-Link PPP (64 kbps)

3 Compressed, Multi-Link PPP  (128 kbps)

It is important to understand that the T/A has to make PPP work

over ISDN *transparently* to the PPP software running on your

computer.  In other words, the T/A itself is acting as an

intermediary, so you can make full use of the T/A with any

already-existing PPP software, without having to modify the

software.

It is absolutely critical that you get a T/A with well-

written firmware, especially where PPP is concerned.  Some T/A's

are so buggy, they barely work with Windows 95's Dial-Up

Networking, and won't work with PPPBBS at all.  In this

event is not the fault of PPPBBS; it is the fault of the

T/A's firmware not correctly supporting standard PPP.

The US Robotics T/A mentioned earlier in this documentation

works great.  All three PPP modes, regular, compressed, and

multilink, have been successfully tested.

PPPBBS UNIQUE ISDN FEATURES

---------------------------

When you make an end-to-end ISDN single B-channel connection

with another computer, host system, or Internet Service Provider,

the connection can be either 64000 bps or 56000 bps, depending on

the capabilities at your phone companies local switching office,

the capabilities of the destination, and all communications

links in between.  Whether you get a good 64000 bps connection

or a 56000 bps connection could vary with time of day, phone

system usage, and will usually vary from call to call.

Whether 64000 or 56000, the call is still digital, and far

superior to an analog modem.

A typical ISDN T/A will work one of two ways:

1) Calls can be forced to 64000

-or-

Calls can be forced to 56000

OR

2) Auto-Sense

-or-

Calls can be forced to 64000

-or-

Calls can be forced to 56000

This is important because sometimes the phone company may

mislabel the actual connect speed of a call.  If your T/A

receives incorrect connect rate information from the phone

company, the call will almost always fail.

Example: You try for an Auto-Sense or Force 64k call.

You actually get a mislabeled 56000 connect.

The T/A will think it is correctly connected,

but no data will be able to pass back and forth.

When you have a T/A connected to PPPBBS Channel 0 for your ISP

dial-out (instead of a plain modem), PPPBBS can be made to

overcome this "ISDN problem".  You can optionally configure

PPPBBS to step through a series of different init strings

until the connection is successful.

When you start-up PPPBBS, and it makes the dial-out to your

ISP, it starts out using the Init String you put on the PPPCFG

main configuration screen, just like if you were using a regular

modem.  The Init String should set the T/A to either "Auto-Sense"

or "Force 64000 bps call" modes.

If the dial-out to your ISP connects, but no data can be passed,

PPPBBS will automatically detect this condition.  It will hang-up

(disconnect) the call, switch to ISDN FallBack Init String #1, and

try the call again.  Ideally, Fall-Back String #1 should set

the T/A for "Force 56000 bps call" mode.

If the dial-out to your ISP connects, but no data can be passed,

PPPBBS will go to FallBack Init String #2, and then to FallBack

Init String #3, if necessary.  Fall-Back #2 might be programmed

to set the T/A to use its built-in analog modem, if it has one.

You may program up to 3 ISDN FallBack Init Strings; you do

not have to program all three to use the feature -- leave

the ones you don't need blank.

US Robotics Courier I-Modem

ISDN Terminal Adapter Init Strings

& FallBack Strings

----------------------------------

Set your primary Init String

to one of these three:

Regular PPP       AT&F1*v2=5*p=1s67=0

Compressed PPP    AT&F1*v2=5*p=2s67=0

Multilink PPP     AT&F1*v2=5*p=3s67=0

Set your FallBack Init Strings to:

FallBk #1:        ATS67=2

FallBk #2:        AT*v2=3

FallBk #3:        (unused -- leave blank)

The primary Init String sets the T/A for

Auto-Sense, and the PPP of your choice.

FallBk #1 tries a forced 56000 call.

FallBk #2 tries an analog modem call using

the Courier I-Modem's built-in modem.

----------------------------------

Anytime that a dial-out is 100% successful, then PPPBBS

reverts back to the primary Init String for the next dial-out.

"Call-Watch" is another ISDN feature.  Set to zero to disable

this feature entirely.  If non-zero, then your PPPBBS will not be

able to make more dial-out attempts per hour than the value set.

The purpose of this feature is simple:

ISDN calls connect quickly -- in one to three seconds.

If there is some type of technical problem (perhaps

at your ISP's end), it is very possible for your ISDN

T/A to make a LOT of "failed" dial-out attempts in a

short period of time.

If you are only allowed to make "so many" outgoing calls

per month on your ISDN line, and have to pay extra for

outgoing calls over a certain amount, then you can see

where the FallBk Init Strings and Call-Watch feature could

save you a lot of money.

Suggested range for the Call-Watch feature, 12 - 24.  Feel

free to adjust this up or down to match your own personal

comfort level.

WHERE CAN I CALL WITH ISDN?

---------------------------

ISDN is totally useless if there is nobody to call with it.

Fortunately today, most ISPs support it.  If your ISP supports

K56flex, X2, or both, then chances are they also support ISDN

on those very same phone numbers.